MS-5 ACPI Controller Change Notice

How to read version form the top marking (Line 2:Part_No (MS-5) + Date Code (XXXX) + Assembly House (x) + IC Version (XRB), the old version is E, new is CRB): MS-5 2208GE, MS-5 2939GCRB

CRB version MS-5 Circuit notice:

Refer above version CRB MS5 circuit, the red circle mark component must be removed and the blue circle mark component must be added. The red/blue circle mark is described as below:

  - Signal SLOT_RST#(Pin4) has be modified to push pull, the pull high resistor R11(18 Ohm) can be removed.

  - The diode D2 can be removed to increase the MS5 chip yield rate

  - New version MS5 has solved the V_DIMM (2.5V) oscillated issue, the diode D3 can be removed.

  - To prevent the MOS turn on abnormally when power on, the CAP C11(102P) must be added

ATX CONN. Circuit notice:

Refer above ATX power connector circuit, if used version CRB MS5, the red circle mark component can be removed and the power connector signal PWR_OK connect to MS5 directly. The red circle mark is described as below:

- New version MS5 doesn’t check signal PWR_OK, these component R763(10K Ohm), R762 (0 Ohm), R761(4.7K Ohm), transistor Q64 & diode D26 can be removed. Signal PWR_OK connect to MS5 directly.

Version change detail

OLD VER.               NEW VER.

DDR Termination Source Voltage Select

- DDR termination source connect to VCC3 or VDIMM is according to chipset design different.

- Please measure below red circle voltage is high or low when S3 state.

Measure point

- If measure point have voltage, the Q45 source voltage from 3VDUAL else VCC3.

- The MS-5 BT_DRV is trace VDIMM voltage when S3 state VDIMM is high then BT_DRV is high, when S4, S5 state VDIMM and BT_DRV is keep low.

Therefore design the termination regulator source voltage, the chipset memory bus high or low status must be confirm when S3 state.